Modbus Master ActiveX Control

Following is a concise user’s manual for the operation of  the Modbus Master ActiveX control

I.	Overview

II.	Modbus Master Control Description

A.	Properties

B.	Methods

C.	Events

- API Reference

IV.	Visual Basic Example

Control Overview

Modbus_master.ocx contains a 32-bit custom control which allows Visual Basic and other OLE Container applications to quickly and easily access data points contained in a modbus slave device connected to the PC.  The modbus device may be connected directly to a PC COM port or accessed via a network connection using the modbus/TCP, communications protocol standard.  Modem connections are also supported using the Win32 TAPI, (Telephony APplication Interface) library.  Multiple connections may be made from the application to different COM ports and/or network devices and each connection operates independently, allowing asynchronous operation to multiple devices on multiple modbus networks.

The Modbus_Master Control exposes properties to define the physical connection, ( such as BaudRate, Parity, etc.),  and methods to connect to a com port, network, or TAPI device.  Each connection is identified by a unique handle returned from the control when the connection is established.  The application initiates messages, (reads & writes), to a device using either the PollModbus and/or WriteModbus method.  All messages are processed by the control in the background allowing the application logic to continue while the character string is formatted and transmitted to the selected device.  The control processes all response messages, verifies the proper checksum and posts the SlaveResponse Event to the application detailing the results of the message transaction.  Upon processing the SlaveResponse Event, the application can obtain the results of the read or write request.

Distribution

The Modbus_master control is distributed by WinTECH Software Design as a time-limited demo control that allows you to freely trial the operation before purchasing a developer license.  The demo control may be integrated with Visual Basic to build modbus master applications that run for 30 minutes at a time.  During this 30-minute trial, the control is fully functional and the application has full access to one or more modbus connections.  At the end of 30 minutes, the unlicensed control will stop communicating with the modbus slaves and the application will have to be restarted to begin another 30-minute communication period.  Purchasing a license for the control will remove all time restrictions and allow the control to be distributed with the application in a run-time environment with no time limits and no additional royalty or licensing payments.

RegSvr32

RegSvr32.exe is a DOS application supplied by Microsoft for making additions and changes to the Windows Registry.  Before the modbus_master control can be used by an application it must be properly installed into the Registry.  To do this, execute RegSvr32.exe and supply the full path name to the control as an argument.  The following command line shows how to register the modbus_master control, (assuming the file is contained in the C:\MODBUS directory):

Regsvr32 c:\modbus\modbus_master.ocx

Modbus Master Control Description

The Modbus_Master Control consists of properties to define the operating characteristics of a physical connection to a modbus network and methods that allow the application to initiate and read response messages from a connected slave device.

The control supports the following modbus message types:

| 01 | READ COIL STATUS |
| --- | --- |
| 02 | READ INPUT STATUS |
| 03 | READ HOLDING REGISTERS |
| 04 | READ INPUT REGISTERS |
| 05 | WRITE SINGLE COIL |
| 06 | WRITE SINGLE REGISTER |
| 15 | WRITE MULTIPLE COILS |
| 16 | WRITE MULTIPLE REGISTERS |

Logic involved with an application accessing data from a modbus slave device using the modbus_master control would be similar to the following:

- Setup the control properties for a connection

- Issue a control method to establish the connection

- Issue a command using one of the above message types

- Wait for the control to post the SlaveResponse Event

- Read the message results

The following properties and methods define the operating characteristics of the Modbus_Master Control:

Properties

| TransmissionMode | MODBUS Transmission Mode (0=ASCII 1=RTU). |  |
| --- | --- | --- |
| TimeOut | Defines the timeout associated with the protocol for this control. Specified in milliseconds, this is the amount of time the MODBUS driver waits for a slave device to respond after polling for data. |  |
| BaudRate | Serial Baud Rate setting. (300, 600, 1200, 2400, 4800, 960014400,19200) |  |
| Parity | Parity selection for COM port connections: (0=None, 1=Odd, 2=Even) |  |
| StopBits | Number of Stop Bits, (0=1, 1=1.5, 2=2). |  |
| DataBits |  | Number of data bits, (7 or 8) |
| TCPDevice |  | Defines the modbus/TCP server for network connections. This may be in the form of an IP Address or Machine Name. This property will be used by the next call to the ConnectModbusTCP() method. |
| PhoneNumber |  | Defines phone number to dial for a modem, (TAPI), connection |

Methods

| ConnectSerial() | Attempts a connection to the modbus via one of the PC’s COM ports. . |
| --- | --- |
| ConnectModbusTCP() | Attempts a network connection to a modbus/TCP Server. . |
| DialTAPIDevice() | Attempts a remote modem connection by dialing the specified TAPI device. |
| PollModbus() | Issues a read request, (modbus message type 01-04), to a designated slave. . |
| WriteModbus() | Issues a write request, (message type 05, 06, 15 or 16). |
| ReadResponse() | Reads the results returned from the slave in response to a PollModbus() or WriteModbus() request |
| Disconnect() | Cleans up the designated connection and frees up resources for use by other Windows applications.. |
| HangupTAPI() | Allows the application to abort a call in progress. . |
| NumberOfTAPIDevices() | This method returns the number of TAPI devices installed on the machine. |
| GetTAPIDeviceName() | This method returns the TAPI Device name for each installed device. (GetTAPIDeviceName() MUST be called for the device prior to issuing the DialTAPIDevice() method). |

Events

The Modbus Master Control posts the following events:

| SlaveResponse | The SlaveResponse Event is fired by the control to signify the completion of a message initiated by the PollModbus() or WriteModbus() method. |
| --- | --- |
| ConnectionEstablished | For network and TAPI connections, this event is fired to signify that the connection is complete. ConnectionEstablished is not fired for serial COM connections. |
| ConnectionDropped | Connection Dropped is fired by the control whenever a TAPI or network connection fails. This event may be fired during the initial connection attempt or anytime thereafter to signify a problem with the connection. |

Modbus Master API Reference

Long ConnectSerial (short Port)

Example:

iHandle = Modbus_Master1.ConnectSerial (1)

(Connect using COM Port 1)

The ConnectSerial() method accepts a single parameter to identify the PC COM port to be used for the connection.  The designated port is opened using the current values for the BaudRate, DataBits, StopBits, and Parity properties. The value returned from ConnectSerial contains the handle to the connection, and must be maintained by the controlling application as a  parameter to be passed to PollModbus(), WriteModbus(), ReadResponse(), etc.  If an error occurs duiring opening of the serial port, the ConnectSerial method returns an INVALID_HANDLE_VALUE, (-1).

Long ConnectModbusTCP (short Port)

Example:

iHandle = Modbus_Master1.ConnectModbusTCP (502)

(Connect to modbus/TCP Port 502)

The ConnectModbusTCP() method instructs the control to establish a modbus/TCP connection to the network device specified by the TCPDevice property using the port number supplied as an argument.  (TCP port number 502 is the standard service port defined by the modbus/TCP protocol standard as published by Modicon.)

ConnectModbusTCP() will return a non-zero, (positive), value if the connection was initiated, however, this DOES NOT confirm that the connection has been established.  The application must process the ConnectionEstablished and ConnectionDropped events to verify that the connection is ready to accept message transactions.

Long DialTAPIDevice (short DeviceIndex)

Example:

iHandle = Modbus_Master1.DialTAPIDevice(0)

(Connect using the first defined TAPI Device)

The DialTAPIDevice() method instructs the control to open a remote TAPI connection by dialing the number specified by the PhoneNumber property.  DialTAPIDevice() will return control immediately to the application while the dial attempt is carried out in the background.  .  An idiosyncrasy of the modbus_master control is that the application must call both NumberOfTAPIDevices() and GetTAPIDeviceName() prior to calling the DialTAPIDevice() method.  Failure to do so will result in the TAPI library not being properly initialized for outgoing calls.

DialTAPIDevice() will return a non-zero, (positive), value if the connection was initiated, however, this DOES NOT confirm that the connection has been established.  The application must process the ConnectionEstablished and ConnectionDropped events to verify that the connection is ready to accept message transactions.

Boolean Disconnect (Long ConnectionHandle)

Example:

Status = Modbus_Master1.Disconnect(iHandle)

(Release all resources associated with the specified Connection Handle)

Disconnect() closes the specified connection and releases all allocated resources for use by other Windows applications.  The return value will be TRUE is the connection is successfully closed

void HangupCall (Long ConnectionHandle)

Example:

Modbus_Master1.HangupCall(iHandle)

(Abort a dial attempt)

HangupCall() should only be used to abort a TAPI connection attempt, (Dial in Progress).  After a TAPI connection has been established, it should be closed using the Disconnect() method.

Long NumberOfTAPIDevices ()

Example:

NumberOfDevices = Modbus_Master1.NumberOfTAPIDevices()

(Determine the number of installed TAPI Devices)

NumberOfTAPIDevices allows the application to interrogate the TAPI library to obtain the number of devices currently installed on the machine.  Generally, the application will display each device name in a drop-down device list to allow the user to select the particular modem to use for the connection.  An idiosyncrasy of the modbus_master control is that the application must call both NumberOfTAPIDevices() and GetTAPIDeviceName() prior to calling the DialTAPIDevice() method.  Failure to do so will result in the TAPI library not being properly initialized for outgoing calls.

String GetTAPIDeviceName (Long DeviceIndex)

Example:

FirstModem = Modbus_Master1.GetTAPIDeviceName(0)

(Get the name of the first TAPI Device)

GetTAPIDeviceName allows the application to interrogate the TAPI library to obtain the name of an installed device.  Generally, the application will display each device name in a drop-down device list to allow the user to select the particular modem to use for the connection.  An idiosyncrasy of the modbus_master control is that the application must call both NumberOfTAPIDevices() and GetTAPIDeviceName() prior to calling the DialTAPIDevice() method.  Failure to do so will result in the TAPI library not being properly initialized for outgoing calls.

Long PollModbus (Long ConnectionHandle, short SlaveNodeAddress, short ModbusFunctionCode, Long Address,  short Length)

Example:

Status = Modbus_Master1.PollModbus (iHandle, 1, 3, 100, 10)

(Read 10 Holding Registers from Node 1 starting at address 100)

# The PollModbus() method initiates a read request to the defined slave.  ConnectionHandle defines the connection and must be equal to the handle returned from ConnectSerial(), DialTAPIDevice() or ConnectModbusTCP();  SlaveNodeAddress defines the modbus slave.  The ModbusFunctionCode must be:

# Function Code		Data Type

# 01			COIL STATUS

# 02			INPUT STATUS

# 03			HOLDING REGISTERS

# 04			INPUT REGISTERS

# Address defines the point address and must be in the range 1-65535.  The Length parameter defines the number of data points to read and must be between 1-256.  If PollModbus() returns zero, the message has been transmitted out the connection and the application will receive a SlaveResponse event later.  (The SlaveResponse event will be fired even if the message times-out.)  If PollModbus() returns a non-zero error code, no SlaveResponse event will be fired.

Long WriteModbus (Long ConnectionHandle, short SlaveNodeAddress, short ModbusFunctionCode, Long Address,  short Length, Long *pData)

Example:

Dim MyData(10) as Long

MyData(0) = 1234

Status = Modbus_Master1.WriteModbus (iHandle, 1, 6, 100, 1, MyData(0))

(Write 1234 to address 100 of Node 1)

# The WritelModbus() method initiates a write request to the defined slave.  ConnectionHandle defines the connection and must be equal to the handle returned from ConnectSerial(), DialTAPIDevice() or ConnectModbusTCP();  SlaveNodeAddress defines the modbus slave.  The ModbusFunctionCode must be:

# Function Code	Data Type

# 05		SINGLE COIL WRITE

# 06		SINGLE REGISTER WRITE

# 15		MULTIPLE COIL WRITE

# 16		MULTIPLE REGISTER WRITE

# Address specifies the data point address and length defines the number of data values to write.

# pData points to an array of long values, (0-65535), to write to the slave.  Refer to the VB example application for additional code samples.

Long ReadResponse (Long ConnectionHandle, short SlaveNodeAddress, short ModbusFunctionCode, Long Address,  short Length, Long *pData)

Example:

Dim MyData(10) as Long

Status = Modbus_Master1.ReadResponse (iHandle, 1, 3, 100, 10, MyData(0))

(Read the response initiated by PollModbus)

# After an application initiates a read or write message to a modbus slave, the modbus_master control will fire the SlaveResponse event when the results of the message are available to be read.  ReadResponse() retrieves the results of the last message transaction for the connection.

# The SlaveNodeAddress, ModbusFunctionCode, Address, and Length parameters must match the last message transmitted for the connection.

# ReadResponse will return a zero error status if the message was successfully transmitted to the slave device and appropriate results obtained.  If the message times out, (or any other error occurs during processing of the message), the appropriate error will be returned.  For a read request, pData will be filled with the data obtained from the slave.  The application must insure that the array pointed to by pData is large enough to contain all the data specified by the Length parameter.  Refer to the VB example for additional code samples.

# Visual Basic Example

An example Visual Basic project is included with the modbus_master distribution zip file.  This example, (Project1), was designed using Visual Basic 6.0 and consists of a single VBA form containing standard controls to allow you to connect, read and write values from/to a modbus slave device.  The example is designed to mirror the api of the modbus_master control with buttons to connect serially, or via a modem, or network.  Edit controls allow you to select a slave node address, read a group of data points and/or write a group of points.

Default settings for the example application operate on COM Port number 1 at 9600 Baud, 8 Data bits, 1 Stop Bit and no parity, using the RTU transmission mode.  The default values for these comm port settings as well as the PhoneNumber and TCPDevice properties used for TAPI and modbus/TCP connections easily customized by accessing the modbus_master property page during design time or programmatically as in the example code.

Operation of the Project1 example begins by opening a connection using one of the three buttons on the left of the form.   When a connection is successful, the Read and Write buttons will be activated.  Pressing the Read Button initiates a PollModbus() method using the defined slave adrdess, point type, point address and length values.

The results of the read request are obtained whenever the modbus_master fires the SlaveResponse event:

‘Global Data Definitions

Dim MyHandle As Long

Dim MyStatus As Integer

Dim Slave_Address As Integer

Dim Command As Integer

Dim Address As Long

Dim Length As Integer

Dim MyData(50) As Long

‘ Generic function to display status value returned

‘ from PollModbus, WriteModbus

Public Sub show_status(ErrCode As Integer)

If (ErrCode = 0) Then

STATUS.Text = "OK"

ElseIf (ErrCode < 255) Then

STATUS.Text = "Slave Device Exception Response"

ElseIf (ErrCode = 256) Then

STATUS.Text = "Invalid Connection Handle"

ElseIf (ErrCode = 257) Then

STATUS.Text = "Message Overrun"

ElseIf (ErrCode = 258) Then

STATUS.Text = "Invalid Point Address"

ElseIf (ErrCode = 259) Then

STATUS.Text = "Invalid Slave Node Address"

ElseIf (ErrCode = 260) Then

STATUS.Text = "Invalid Length"

ElseIf (ErrCode = 261) Then

STATUS.Text = "Unsupported Modbus Command"

ElseIf (ErrCode = 263) Then

STATUS.Text = "Slave Device Time-Out"

ElseIf (ErrCode = 264) Then

STATUS.Text = "Invalid Transmission Mode"

ElseIf (ErrCode = 265) Then

STATUS.Text = "Invalid CRC In Slave Response"

ElseIf (ErrCode = 266) Then

STATUS.Text = "Connection Not Established"

ElseIf (ErrCode = 267) Then

STATUS.Text = "Invalid Slave Response"

ElseIf (ErrCode = 271) Then

STATUS.Text = "Demo Time Expired"

ElseIf (ErrCode = 272) Then

STATUS.Text = "Invalid modbus/TCP Command"

End If

End Sub

‘ Handler for the CONNECT SERIAL Button

Private Sub ConnectSerial_Click()

Modbus_master1.BaudRate = 9600 	 '9600 Baud

Modbus_master1.Parity = 0       		'NoParity

Modbus_master1.DataBits = 8     		'8 DataBits

Modbus_master1.StopBits = 0    	 	'One Stop Bit

Modbus_master1.TimeOut = 2000  		 '2000 msec TimeOut

Modbus_master1.TransmissionMode = 1 	'RTU

MyHandle = Modbus_master1.ConnectSerial(1)  ' Connect to COMM Port 1

If MyHandle > 0 Then

‘ Connection was successful

‘ (This example only allows a single connection)

‘ Disable All Connection Buttons

‘ Enable the Read, Write & Disconnect Buttons

ConnectSerial.Enabled = False

ConnectTAPI.Enabled = False

ConnectTCP.Enabled = False

Disconnect.Enabled = True

STATUS.Text = "Connected"

READMODBUS.Enabled = True

WRITEMODBUS.Enabled = True

Else

‘Connection Attempt Failed

‘(Another application must have control of the COM Port)

STATUS.Text = "Not Connected"

End If

End Sub

‘ Handler for the CONNECT TAPI Button

Private Sub ConnectTAPI_Click()

‘Go through the motions of getting the TAPI Device List

nTAPIDevices = Modbus_master1.NumberOfTAPIDevices()

TAPIDevice = Modbus_master1.GetTAPIDeviceName(0)

‘Setup the phone number to dial

Modbus_master1.PhoneNumber = "645-5966"

‘Dial the call

MyHandle = Modbus_master1.DialTAPIDevice(0)

If MyHandle > 0 Then

‘Call should be in progress now

‘Don’t enable the Read & Write Buttons

‘until we get the CallEstablished Event

STATUS.Text = "Connecting"

ConnectSerial.Enabled = False

ConnectTAPI.Enabled = False

ConnectTCP.Enabled = False

Disconnect.Enabled = False

READMODBUS.Enabled = False

WRITEMODBUS.Enabled = False

Else

STATUS.Text = "Not Connected"

End If

End Sub

‘ Handler for the CONNECT TCP Button

Private Sub ConnectTCP_Click()

‘ Select the Device to connec tto

‘ In this case use the IP Loopback address to

‘ connect to the local machine

Modbus_master1.TCPDevice = "127.0.0.1"

MyHandle = Modbus_master1.ConnectModbusTCP(502)

If MyHandle > 0 Then

‘Connection should be in progress now

‘Don’t enable the Read & Write Buttons

‘until we get the CallEstablished Event

STATUS.Text = "Connecting"

ConnectSerial.Enabled = False

ConnectTAPI.Enabled = False

ConnectTCP.Enabled = False

Disconnect.Enabled = False

READMODBUS.Enabled = False

WRITEMODBUS.Enabled = False

Else

STATUS.Text = "Not Connected"

End If

End Sub

‘ ConnectionEstablished Event Handler

‘ Initiated from either ConnectModbusTCP() or DialTAPIDevice()

Private Sub Modbus_master1_ConnectionEstablished(ByVal hConnect As Long)

‘Enable the Disconnect Button

‘Enable Read & Write Buttons

ConnectSerial.Enabled = False

ConnectTAPI.Enabled = False

ConnectTCP.Enabled = False

Disconnect.Enabled = True

STATUS.Text = "Connected"

READMODBUS.Enabled = True

WRITEMODBUS.Enabled = True

End Sub

‘ ConnectionDropped Event Handler

Private Sub Modbus_master1_ConnectionDropped(ByVal hConnect As Long)

‘Either the TCP or TAPI connection attempt failed

‘or something has happened to abort the connection after it

‘has been established for a while.

‘In either case we’re now disconnected so enable the

‘buttons accordingly

MyHandle = -1

ConnectSerial.Enabled = True

ConnectTAPI.Enabled = True

ConnectTCP.Enabled = True

Disconnect.Enabled = False

STATUS.Text = "Not Connected"

READMODBUS.Enabled = False

WRITEMODBUS.Enabled = False

End Sub

‘ Handler for the DISCONNECT Button

Private Sub Disconnect_Click()

‘Tell the control to Disconnect

‘ReEnable the Connect Buttons for new connection attempt

MyStatus = Modbus_master1.Disconnect(MyHandle)

MyHandle = -1

ConnectSerial.Enabled = True

ConnectTAPI.Enabled = True

ConnectTCP.Enabled = True

Disconnect.Enabled = False

STATUS.Text = "Not Connected"

READMODBUS.Enabled = False

WRITEMODBUS.Enabled = False

End Sub

‘ Handler for the READ Button

Private Sub READMODBUS_Click()

‘Get the Slave Node Address, Modbus Command, Address & Length

‘ from the appropriate Edit controls

Slave_Address = NODEADDRESS.Text

Command = POINTTYPE.Text

Address = READADDRESS.Text

Length = READLENGTH.Text

‘We must remember these parameters so we can use

‘them in the ReadResponse method to make sure we

‘get what we ask for

‘

‘Initiate the Read Request

MyStatus = Modbus_master1.PollModbus(MyHandle, Slave_Address, Command, Address, 					Length)

‘Check the status to make sure the request went out.

If MyStatus = 0 Then

STATUS.Text = "Busy"

Else

show_status (MyStatus)

End If

End Sub

‘ Handler for the WRITE Button

Private Sub WRITEMODBUS_Click()

‘Get the Slave Node Address, Modbus Command, Address & Length

‘ from the appropriate Edit controls

Slave_Address = NODEADDRESS.Text

Address = WRITEADDRESS.Text

Length = WRITELENGTH.Text

‘Make sure we do not overrun the defined data array

If Length > 50 Then

Length = 50

End If

‘ Write a single pattern to all points

For i = 0 To Length - 1

MyData(i) = PATTERN.Text

Next i

‘Check to see which modbus function code

‘ to use (05 for single coil write, 15 for multiple coils,

’06 for single register, 16 for multiples)

If POINTTYPE.Text < 2 Then

If Length = 1 Then

Command = 5

Else

Command = 15

End If

Else

If Length = 1 Then

Command = 6

Else

Command = 16

End If

End If

‘ Initiate the Write Request Message

MyStatus = Modbus_master1.WRITEMODBUS(MyHandle, Slave_Address, Command, Address, 							Length, MyData(0))

If MyStatus = 0 Then

STATUS.Text = "Busy"

Else

show_status (MyStatus)

End If

End Sub

‘Process the Slave Response Message

‘Modbus_Master SlaveResponse Event Handler

Private Sub Modbus_master1_SlaveResponse(ByVal hConnect As Long)

‘In a multi-connection environment

‘hConnect will identify which connection this event represents

‘

‘Read the results of the last message request

MyStatus = Modbus_master1.ReadResponse(hConnect, Slave_Address, Command, Address, 				Length, MyData(0))

‘Update the Status display

show_status (MyStatus)

‘If the last message was from PollModbus, (i.e. Read Request)

‘Update the modbus data display on the form

If Command <= 4 Then

‘If the response was received with no errors

‘Update the text controls with data obtained

‘ from the slave

If MyStatus = 0 Then

For i = 0 To 49

If i < Length Then

Text1(i).Text = MyData(i)

Else

Text1(i).Text = "?"

End If

Next i

Else

‘Otherwise indicate that the read data is unknown

For i = 0 To 49

Text1(i).Text = "?"

Next i

End If

End If

End Sub